TECHNICAL HANDOFF DOCUMENT

An Agentic Multi-Model Architecture for Proactive Safety in Autonomous Transportation Systems

ASCE2027: The Infrastructure and Engineering Experience

Submission ID: ASCE2027-1804  |  Philadelphia, March 1-5, 2027

Prepared by: Joyjit Roy  |  For: Samaresh Kumar Singh (Research Partner)

Date: June 2026

# 1. Purpose of This Document

This document provides complete technical instructions for Samaresh Kumar Singh to implement the proof-of-concept pipeline for the ASCE2027 proceedings paper. It covers the final architecture decisions, model choices with rationale, dataset specifications with exact field schemas, file structure, coding instructions, and deadlines.

The paper extends SafeDriver-IQ (Phase 1, IEEE EIT 2026) into a three-component agentic architecture for proactive transportation safety. The primary contribution is the architecture design. The proof-of-concept validates feasibility using existing and new datasets.

# 2. Research Context and Paper Summary

## 2.1 Phase 1 Foundation: SafeDriver-IQ

SafeDriver-IQ (published IEEE Access Q1, presented IEEE EIT 2026) introduced inverse crash probability modeling. It converts binary crash classifiers into continuous 0-100 driver safety scores. Key results:

- Trained on 213,003 NHTSA CRSS crash records (2016-2023)

- Validated on 500 Waymo Open Motion Dataset scenarios

- Random Forest model, inference under 1ms per sample

- 87% of crashes involve multiple co-occurring risk factors

- VRU + Urban + Night combination reaches 4.5x baseline crash risk

- Estimated 22.7% crash reduction at moderate ADAS threshold

- SHAP and ablation alignment at r=0.94

- GitHub: https://github.com/ssam18/safedriver-iq

- arXiv: https://arxiv.org/abs/2603.14841

## 2.2 Known Limitation SafeDriver-IQ Must Address

The SafeDriver-IQ GitHub README explicitly documents that the current model does NOT respond meaningfully to VRU presence or road surface changes. It only captures environmental context (weather, lighting, time of day). The ASCE2027 agentic architecture directly addresses this limitation by adding dedicated trajectory and VRU components on top of SafeDriver-IQ.

## 2.3 Phase 2 Paper: What We Are Building

The ASCE2027 paper presents an agentic multi-model safety architecture that shifts from reactive crash avoidance to proactive, continuous risk management. The architecture is the primary contribution. The proof-of-concept validates that the components work on real data.

# 3. Final Architecture Design

## 3.1 Architecture Overview

3.2 Architecture Diagram and Description

Fig. 1. Agentic Multi-Model Safety Architecture. Environmental risk scoring (SafeDriver-IQ) acts as a context multiplier for parallel trajectory kinematic and VRU interaction models. An RL agent with short and long-term memory fuses all signals into graduated safety interventions.

Layer 1: Data Inputs

- NHTSA CRSS (2016–2023, 213,003 crash records) — national crash statistics that define what dangerous driving looks like

- Waymo WOMD (full validation corpus) — real-world autonomous vehicle trajectories

- Argoverse 2 (24,988 scenarios) — vehicle and pedestrian movement data across 6 US cities

- nuScenes mini (10 scenes) — multimodal sensor data captured specifically under rain and night conditions

Layer 2: Three Parallel Models (running simultaneously)

- Environmental Risk Scoring (SafeDriver-IQ, Random Forest) — scores how dangerous the current environment is on a scale of 0 to 100. This score acts as a context multiplier, amplifying the other two signals when conditions are already dangerous

- Trajectory Kinematic Analysis (LSTM) — analyzes how the vehicle is moving, including speed, acceleration, heading change, and trajectory pattern over time

- VRU Interaction Prediction (Social Force Model + LSTM) — watches pedestrians and cyclists, predicts their movement, and flags proximity risks including near-miss situations

Layer 3: Agentic Reasoning Layer

- RL Agent — receives all three signals simultaneously and decides the appropriate intervention

- SHAP Explainability — explains which factors drove each decision

- Short-term memory — tracks what has happened in the current drive

- Long-term memory — applies lessons learned from thousands of training scenarios

- Graduated output — silent at score 70–100, advisory at 40–70, specific intervention at 20–40, emergency alert below 20

Layer 4: Applications

- ADAS Integration, Fleet Risk Management, Infrastructure Planning

The architecture has four components operating together in real time. Three specialized models run in parallel. A central agentic reasoning layer fuses their outputs and produces graduated safety interventions.

| Component | Model | Primary Dataset | Role |
| --- | --- | --- | --- |
| Environmental Risk Scoring | Random Forest (SafeDriver-IQ) | NHTSA CRSS | Scores ambient environment 0-100 |
| Trajectory Kinematic Analysis | LSTM | Waymo Full + Argoverse 2 val | Analyzes vehicle motion patterns |
| VRU Interaction Prediction | Social Force Model + LSTM hybrid | nuScenes mini + Argoverse 2 val | Predicts pedestrian/cyclist behavior |
| Agentic Reasoning Layer | RL Agent + SHAP | All three datasets | Fuses signals, produces interventions |

## 3.3 Key Architecture Decisions

Decision 1: SafeDriver-IQ as Context Multiplier (not equal input)

SafeDriver-IQ does not contribute as one equal signal among three. It acts as a context multiplier that adjusts how the trajectory and VRU models weight their outputs. Environmental conditions amplify everything else. A pedestrian near a vehicle at night in rain is a fundamentally different risk than the same scenario in clear daylight. SafeDriver-IQ sets the risk environment that all other components operate within.

Decision 2: All Three Models Run in Parallel

All three models run simultaneously every decision cycle. The agentic layer does not decide which models to activate. This is a deliberate safety design choice. The core finding from SafeDriver-IQ is that 87% of crashes involve multiple co-occurring factors. Sequential or conditional model activation creates blind spots. The agentic layer's role is to determine how much weight to assign each signal given current context, not to decide whether to listen to them.

Decision 3: Graduated Output (Score + Intervention)

The agentic layer produces both a unified safety score and a tiered intervention based on that score. This extends SafeDriver-IQ's existing threshold table:

| Score Range | Risk Level | Output Type | Example |
| --- | --- | --- | --- |
| 70-100 | Low/Safe | Score only, no alert | Safety: 82/100 |
| 40-70 | Moderate | Score + advisory guidance | Score 58: reduce speed, pedestrian risk ahead |
| 20-40 | High | Score + specific intervention | Score 31: slow to 20mph, cyclist approaching left |
| 0-20 | Critical | Emergency intervention | Score 12: brake now, collision risk |

## 3.4 Component Details

Component 1: Environmental Risk Scoring (SafeDriver-IQ)

This component is already built. Use the existing Random Forest model from the SafeDriver-IQ repository. Do not retrain. Load the saved model from results/crash_investigation_rf_model.pkl. Input is the 64-feature vector from NHTSA-derived environmental features. Output is the 0-100 safety score which becomes the context multiplier for all other components.

Component 2: Trajectory-Based Kinematic Analysis (LSTM)

Model type: LSTM (Long Short-Term Memory). This model processes sequential vehicle movement data at 10Hz and learns temporal patterns associated with elevated crash risk. Key features to extract and feed:

- Speed magnitude derived from velocity_x and velocity_y

- Heading in radians

- Heading change rate (angular velocity proxy)

- Acceleration derived from velocity differences across timesteps

- Trajectory length over the scenario window

- Multi-agent context from SCORED_TRACK and UNSCORED_TRACK categories

The LSTM receives a sequence of these features over time and outputs a trajectory risk score. For the POC, a shallow LSTM (2 layers, 64 hidden units) is sufficient. No full training required at this stage.

Component 3: VRU Interaction Prediction (Social Force Model + LSTM)

This is a two-layer hybrid model. The Social Force Model (SFM) is a physics-based mathematical model (Helbing and Molnar, 1995) that treats pedestrians as particles subject to repulsive forces from vehicles and other agents and attractive forces toward their destination. No training is required for SFM. It runs on mathematical equations.

The LSTM correction layer sits on top of SFM and learns patterns from nuScenes and Argoverse 2 data that pure physics misses. For the POC, implement SFM to predict pedestrian/cyclist base trajectories, then apply a lightweight LSTM to adjust predictions based on learned patterns. Key features:

- Vehicle-to-pedestrian minimum distance over time

- Vehicle-to-cyclist minimum distance over time

- Relative velocity between ego vehicle and nearest VRU

- Time-to-collision proxy where derivable

- Near-miss flag when minimum distance falls below threshold

Component 4: Agentic Reasoning Layer (RL Agent + SHAP)

Model type: Reinforcement Learning agent. The RL agent receives the outputs from all three components and learns the optimal intervention policy. State space: environmental score from SafeDriver-IQ, trajectory risk from LSTM, VRU risk from SFM+LSTM hybrid. Action space: one of four intervention levels (silent, advisory, specific guidance, emergency). Reward: interventions that reduce subsequent risk scores are rewarded; unnecessary interventions are penalized.

SHAP explanations are computed for every intervention output, following the same approach used in SafeDriver-IQ. This ensures each intervention includes feature-level attribution explaining which factors drove the decision. This is critical for the explainability argument in the paper.

For the POC, a simplified RL agent using Q-learning or a lightweight policy network is sufficient. Full deep RL training is not required.

# 4. Datasets: Full Specification

## 4.1 Dataset Overview

| Dataset | Role | Scale | Format | License |
| --- | --- | --- | --- | --- |
| NHTSA CRSS 2016-2023 | Environmental risk training | 213,003 crash records | CSV (already in repo) | Public domain |
| Waymo Open Motion Dataset v1.2 | AV trajectory validation + existing foundation | 500 scenarios in repo; full corpus 100k+ segments | TFRecord (already loaded via waymo_data_loader.py) | Research use |
| Argoverse 2 Motion Forecasting (val split) | Trajectory kinematic analysis, VRU trajectories | 24,988 scenarios, ~6GB compressed | Parquet files per scenario | CC BY-NC-SA 4.0 |
| nuScenes v1.0-mini | VRU multimodal sensing, adverse conditions | 10 scenes, 404 samples, ~4GB | JSON tables + sensor files | CC BY-NC-SA 4.0 |

## 4.2 Waymo Open Motion Dataset

The existing SafeDriver-IQ repo already has waymo_data_loader.py which parses TFRecord format and extracts per-agent states. The current implementation uses 500 scenarios from a single shard. For the ASCE2027 paper, scale this to the full validation split.

Dataset structure in repo:

- Location: waymo/motion_dataset/

- Splits: training (1,000 shards), validation, validation_interactive, testing, testing_interactive

- 91 timesteps per scenario at 10Hz (1s context + 8s future horizon)

- Agent types: vehicles, pedestrians, cyclists

- Includes: road graph, traffic signals, speed limits

Existing loader functions to build on:

- waymo_data_loader.py: parse TFRecords, extract agent states, compute TTC, min inter-agent distance, near-miss flags

- Extend this loader to feed the new LSTM trajectory component

Scale target for paper: report results on the full validation split, not just 500 scenarios. This directly addresses the limitation acknowledged in the SafeDriver-IQ paper.

## 4.3 Argoverse 2 Motion Forecasting Dataset

Download instructions:

- Official source: https://www.argoverse.org/av2.html

- AWS S3 bucket: s3://argoverse/datasets/av2/

- Use s5cmd tool or the official av2 Python API to download only the validation split

- Validation split: approximately 24,988 scenarios, 6.2GB compressed

- Extract to: data/argoverse2/val/ in the safedriver-iq repo

File structure per scenario:

- Each scenario is a folder named by scenario_id

- scenario_{scenario_id}.parquet: trajectory data

- log_map_archive_{scenario_id}.json: HD map data

Exact schema: ArgoverseScenario

| Field | Type | Description |
| --- | --- | --- |
| scenario_id | str | Unique scenario identifier |
| timestamps_ns | NDArrayInt | All timestamps in nanoseconds |
| tracks | List[Track] | All agent tracks in this scenario |
| focal_track_id | str | Primary agent track ID |
| city_name | str | One of: Austin, Detroit, Miami, Pittsburgh, Palo Alto, Washington DC |
| map_id | Optional[int] | Map identifier for bookkeeping |
| slice_id | Optional[str] | Slice identifier for bookkeeping |

Exact schema: Track

| Field | Type | Description |
| --- | --- | --- |
| track_id | str | Unique track identifier |
| object_type | ObjectType enum | vehicle, pedestrian, motorcyclist, cyclist, bus, static, background, construction, riderless_bicycle, unknown |
| category | TrackCategory enum | TRACK_FRAGMENT, UNSCORED_TRACK, SCORED_TRACK, FOCAL_TRACK |
| object_states | List[ObjectState] | One entry per observed timestep |

Exact schema: ObjectState (per agent per timestep)

| Field | Type | Description | Use In Architecture |
| --- | --- | --- | --- |
| observed | bool | True if in observed segment (not future) | Filter to observed only for training |
| timestep | int | Time step index [0, num_timesteps) | Sequence ordering for LSTM |
| position | Tuple[float, float] | (x, y) center coordinates in meters | Trajectory path, distance computation |
| heading | float | Heading in radians, map coordinate frame | Heading change rate, lane following |
| velocity | Tuple[float, float] | (vx, vy) instantaneous velocity in m/s | Speed, acceleration (derived), relative velocity to VRU |

Derived features to compute from raw fields:

- Speed magnitude: sqrt(vx^2 + vy^2)

- Acceleration: delta_speed / delta_time across consecutive timesteps

- Heading change rate: delta_heading / delta_time

- Trajectory length: cumulative sum of position deltas

- Vehicle-to-VRU distance: Euclidean distance between vehicle position and each pedestrian/cyclist position

- Relative velocity: difference of velocity vectors between vehicle and VRU

- Time-to-collision proxy: distance / relative_speed where approaching

- Near-miss flag: distance < threshold (suggest 2.0m for pedestrians, 1.5m for cyclists)

Python loading example:

from av2.datasets.motion_forecasting import scenario_serialization

from av2.datasets.motion_forecasting.data_schema import ObjectType, TrackCategory

scenario = scenario_serialization.load_argoverse_scenario_parquet(scenario_path)

vehicles = [t for t in scenario.tracks if t.object_type == ObjectType.VEHICLE]

pedestrians = [t for t in scenario.tracks if t.object_type == ObjectType.PEDESTRIAN]

cyclists = [t for t in scenario.tracks if t.object_type == ObjectType.CYCLIST]

## 4.4 nuScenes v1.0-mini Dataset

Download instructions:

- Official source: https://www.nuscenes.org/nuscenes

- Free registration required at nuscenes.org

- Download link for mini split: https://www.nuscenes.org/data/v1.0-mini.tgz

- Size: approximately 3.9GB compressed

- Extract to: data/nuscenes/ in the safedriver-iq repo

- Install devkit: pip install nuscenes-devkit

Mini split contents:

- 10 scenes, 20 seconds each

- 404 annotated keyframe samples

- Sensor channels: CAM_FRONT, CAM_FRONT_LEFT, CAM_FRONT_RIGHT, CAM_BACK, CAM_BACK_LEFT, CAM_BACK_RIGHT, LIDAR_TOP, RADAR_FRONT, RADAR_FRONT_LEFT, RADAR_FRONT_RIGHT, RADAR_BACK_LEFT, RADAR_BACK_RIGHT

- Locations: Boston and Singapore

- Explicitly includes night and rain scenes

Core schema tables (all linked by token):

| Table | Key Fields | Relevance |
| --- | --- | --- |
| scene | token, name, first_sample_token, last_sample_token | Top-level 20s driving sequence |
| sample | token, timestamp (microseconds), scene_token | Annotated keyframe at 2Hz |
| sample_annotation | token, sample_token, instance_token, category_token, attribute_tokens, translation [x,y,z], size [w,l,h], rotation [quaternion], num_lidar_pts, num_radar_pts, visibility_token | Per-object annotation per keyframe |
| ego_pose | token, timestamp, translation [x,y,z], rotation [quaternion] | Ego vehicle position at every timestamp |
| instance | token, category_token, nbr_annotations, first_annotation_token, last_annotation_token | Unique object tracked across keyframes |

Object categories for VRU research:

| Category Name | Type | Attributes Available |
| --- | --- | --- |
| human.pedestrian.adult | VRU | pedestrian.moving, pedestrian.standing, pedestrian.sitting_lying_down |
| human.pedestrian.child | VRU | pedestrian.moving, pedestrian.standing, pedestrian.sitting_lying_down |
| human.pedestrian.wheelchair | VRU | pedestrian.moving, pedestrian.standing |
| vehicle.bicycle | VRU | cycle.with_rider, cycle.without_rider |
| vehicle.motorcycle | VRU | cycle.with_rider, cycle.without_rider, vehicle.moving, vehicle.stopped, vehicle.parked |
| vehicle.car | Vehicle | vehicle.moving, vehicle.stopped, vehicle.parked |
| vehicle.bus | Vehicle | vehicle.moving, vehicle.stopped, vehicle.parked |

Python loading example:

from nuscenes.nuscenes import NuScenes

nusc = NuScenes(version='v1.0-mini', dataroot='data/nuscenes', verbose=True)

# Get all pedestrian annotations

ped_anns = [nusc.get('sample_annotation', t) for t in nusc.field2token('sample_annotation', 'category_token', nusc.field2token('category', 'name', 'human.pedestrian.adult')[0])]

# 5. Proof-of-Concept Pipeline: Code Structure

## 5.1 Modular File Structure

Build the POC as a clean modular pipeline. Each module maps directly to a component in the paper architecture. Do not put everything in one script.

| File | Purpose | Inputs | Outputs |
| --- | --- | --- | --- |
| data_loader.py | Unified loader for all datasets | File paths for AV2, nuScenes, Waymo | Standardized scenario objects |
| kinematic_features.py | LSTM trajectory component | AV2 and Waymo vehicle tracks | Per-scenario kinematic feature vectors |
| vru_features.py | SFM + LSTM VRU component | AV2 and nuScenes VRU tracks | Per-scenario VRU interaction features |
| scenario_summary.py | Summary table generator | All feature vectors | DataFrame with one row per scenario |
| safedriver_iq_bridge.py | Bridge to existing SafeDriver-IQ model | Environmental context features | 0-100 safety score (context multiplier) |
| agentic_layer.py | RL agent + SHAP fusion | All component outputs | Graduated intervention + explanation |
| main.py | Full pipeline orchestrator | Dataset paths | Final results table and visualizations |

## 5.2 Output Requirements for the Paper

The paper needs the following results to populate tables and figures. These are the minimum outputs the POC must produce:

- Per-scenario summary table: scenario_id, city, num_vehicles, num_pedestrians, num_cyclists, mean_speed, max_speed, min_vru_distance, near_miss_flag, safedriver_iq_score, final_safety_score, intervention_level

- Distribution plots: safety score distribution across scenarios, near-miss rate by city, VRU distance histogram

- Ablation result: show how score changes when SafeDriver-IQ context multiplier is applied vs not applied

- SHAP feature attribution: top 10 features driving intervention decisions across the validation set

- Waymo scenario validation (extending existing results): mean crash probability by scenario type using full validation split

## 5.3 What NOT to Build for POC

Do not attempt the following for the POC. These are future work items explicitly acknowledged in the paper:

- Full RL agent training on millions of scenarios

- Real-time inference pipeline or live dashboard

- Integration with live sensor streams or OBD-II

- Federated learning (this is Phase 3, not Phase 2)

- Full nuScenes dataset (mini split is sufficient)

# 6. Repository Structure Updates

Add the following to the existing safedriver-iq GitHub repository. Follow the same Git LFS pattern already established for Waymo data.

| New Path | Contents | Notes |
| --- | --- | --- |
| data/argoverse2/val/ | Argoverse 2 validation split parquet files | Store via Git LFS, ~6GB |
| data/nuscenes/ | nuScenes mini split extracted files | Store via Git LFS, ~4GB |
| tests/test_av2_val.py | AV2 validation data loading test | Already written, move here |
| tests/test_nuscenes_mini.py | nuScenes mini loading test | Already written, move here |
| src/data_loader.py | Unified dataset loader (new) | Replaces or extends existing data_loader.py |
| src/kinematic_features.py | LSTM trajectory features (new) | New module |
| src/vru_features.py | SFM + LSTM VRU features (new) | New module |
| src/scenario_summary.py | Summary table generator (new) | New module |
| src/safedriver_iq_bridge.py | Bridge to existing RF model (new) | New module |
| src/agentic_layer.py | RL Agent + SHAP fusion (new) | New module |
| main.py | Full pipeline orchestrator (new) | Top-level entry point |

# 7. Industry Context and Feature Thresholds (Arity Report 2025)

The Arity Annual Driving Behavior Report 2025 analyzes more than 45 million anonymized active U.S. drivers covering more than two trillion miles from 2022 through June 2025. It provides industry-validated behavioral thresholds and contextual insights that directly support the feature engineering in this proof-of-concept pipeline. Use this report as industry motivation and feature-engineering support only. The underlying Arity dataset is proprietary and must not be used as validation data. Cite it as an industry report, not a peer-reviewed source.

7.1 Industry-Validated Feature Thresholds

Apply these thresholds when extracting kinematic features from Argoverse 2 and nuScenes. They are derived from the Arity report and represent industry-standard behavioral risk definitions.

- Sudden acceleration flag: speed increase >= 8 mph per second (3.576 m/s^2). Set binary flag = 1 when threshold exceeded.

- Hard braking flag: speed decrease >= 8 mph per second (3.576 m/s^2). Set binary flag = 1 when threshold exceeded.

- High-speed flag: speed magnitude > 35.76 m/s (80 mph). Set binary flag = 1 when exceeded.

- Stop-and-go congestion flag: count of alternating sudden acceleration and hard braking events within a 30-second window >= 2.

- Distracted driving proxy: not directly derivable from Argoverse 2 or nuScenes. Note as a limitation. In real deployment this would come from phone sensor data at speeds >= 10 mph.

7.2 Emergency Event Argument for Agentic Memory

The Arity report provides two real-world emergency case studies that directly support the need for contextual memory in the agentic reasoning layer. These should be referenced in the paper to motivate the memory component.

- LA Wildfire 2025: driving speeds dropped 17% on the day of the fire; hard braking increased 25.3% during the 10am-1pm evacuation window; phone usage increased 29.1%. A static threshold-based system would not detect this as an emergency pattern. An RL agent with short-term memory tracking the trajectory of these indicators over time would escalate interventions appropriately.

- Hurricane Helene: trip volume increased 30% two days before landfall then dropped 64% on landfall day. Long-distance trips increased. Speed and acceleration patterns did not change significantly. This shows that volumetric and trip-type signals can be early warning indicators even when kinematic signals appear normal.

7.3 Citation Format

Use this citation in the paper references section (ASCE author-date format):

Arity. 2025. Annual Driving Behavior Report 2025. Arity, Chicago, IL. Tech. Rep.

Do not cite Arity as peer-reviewed evidence. It is industry motivation only. All quantitative validation in the paper must come from NHTSA CRSS, Waymo, Argoverse 2, and nuScenes.

ASCE2027-1804  |  SafeDriver-IQ Phase 2  |  Confidential Research Document